WP54 G /  AG series – mayour functions and benefits :

Single-SSID Firmware

| Wireless Operating Mode: | Security Method |
| --- | --- |
| 1: Access Point | 1: MAC Address Filtering |
| 2: Client mode | 2: 64 / 128-bit WEP |
| 3: Point to Point | 3: 802.1x authentication |
| 4: Point to Multiple Point | 4: WPA-EAP and WPA-PSK, WPA2 |
| 5: Wireless Routing CLient |  |
| 6: Gateway |  |
| 7: Wireless Adapter |  |
| Multiple-SSID Firmware |  |
| Wireless Operating Mode: | Security Method |
| 1: Access Point | 1: Station Isolation |
| 2: Client Mode | 2: MAC Address Filtering |
| 3: RootAP / Transparent Client | 3: 64 / 128-bit WEP |
| 4: Repeater | 4: 802.1x authentication |
| 5: Wireless Adapter | 5: WPA-EAP and WPA-PSK, WPA2 |
| 6: Wireless Routing Client |  |
| 7: Gateway |  |
| Common Features |  |
| 1: HTTP / HTTPs Web Server |  |
| 2: SNMP |  |
| 3: Telnet / Secure Shell (SSH) |  |
| 4: Event Logging |  |
| 5: Long Range Parameter Settings |  |
| 6: Spanning Tree Protocol |  |
| 7: Parallel Broadband (only in Routing mode) |  |
| Multiple-SSID |  |
| Supports up to 16 virtual (WP18) or 4 virtual (WP54) access points per radio, with unique BSSIDs. Traffic from each VAP can be tagged to a unique VLAN and /or bridged if required. Each VAP will be able to configure their own security (WEP, WPA, WPA2) |  |
| Long Range Support |  |
| Supports for long-range wireless deployment with Proprietary Long Distance Algorithm for ACK and CTS timeout adjustment support. |  |
| Provides recommended values for the parameters and at the same time allows for manual fine-tuning for optimal performance. |  |
| Antenna Alignment |  |
| Antenna Alignment allows the user to adjust their antenna to receive the optimum throughput. |  |
| Bandwidth Control (Available in Routing Mode) |  |
| Give the administrator the choice to manage the bandwidth control of subscribers in case of massive data transfer which causes slowdown problems when surfing the internet. |  |
| Ability to limit the Upload/Download bandwidth at WAN/LAN ports using either IP address or MAC address. |  |
| Wireless Distribution System (WDS) |  |
| Using the RootAP and Transparent Client combination, access points can be connected together, using MAC address /ESSID. |  |
| Allows RootAPs to be connected to another RootAPs, which in turn can be connected to another RootAP through the WDS Setting (coming soon) |  |
| Spanning Tree Protocol |  |
| Provides extra redundancy and able to auto-reconfigurable when there are changes in the network topology |  |
| IEEE 802.11h (DFS & TPC) |  |
| Enables worldwide operation through support for standards-based Dynamic Frequency Selection (DFS) and Transmission Power Control (TPC) |  |
| DHCP Relay (Only in Router Mode) |  |
| Allows DHCP Clients on different subnets to get IP address from central DHCP server |  |
| Antenna Control |  |
| Allows user to select which antenna connector on the radio to use, Options are either MAIN/AUX/Diversity. |  |
| SNMP Trap |  |
| SNMP Traps enable an agent to notify the management station of significant events by way of an unsolicited event. |  |
| Preferred AP List |  |
| Users have the opportunity to input 4 MAC addresses of the APs that they prefer to connect to under all Client modes. |  |
| Remote Upgrade of Firmware |  |
| Allows user to upgrade their firmware through Telnet/SSH |  |
| RIP 1/2 (Only in Routing Mode) |  |
| Routing Information Protocol version 1/2. |  |
| 4.9GHz Public Safety Spectrum Supported (Multiple-SSID Firmware) |  |
| WP54 and WP18 with Compex WLM54AG miniPCI supported the 4.9GHz spectrum specifically for homeland security and public safety related operation. |  |
| Variable 5, 10 and 20MHz Transmit Bandwidth. |  |
| Certain of the features maybe not available in some firmware. please check the datasheet or contact us. |  |
| Please refer to the datasheet of the corresponding model for more advanced features. |  |

List of all COMPEX´s unique features !!