Catalogue- Comand Lined description

2.Homeplug EoC AV74 command	7

2.1 show	7

2.1.1. show cbat_info	7

2.1.2. show cbat_topo_info	8

2.1.3. show cbat_perf	9

2.1.4 show cbat_vlan_list	9

2.1.5 show cbat_vlan_info (vlanid)	9

2.1.6 show card_info	10

2.1.7 show card_topo_info	10

2.1.8 show card_rf_info	11

2.1.9 show card_qos	12

2.1.10 show analog_info	12

2.1.11 show cnu_info	13

2.1.12 show cnu_qos	15

2.1.13 show cnu_rt_stats	16

2.1.14 show default_cnu_config	16

2.1.15 show mgmt_vlan	18

2.1.16 show pre_cnu_list	18

2.1.17 show pre_cnu_config	19

2.1.18 show trap_server	20

2.1.19 show whitelist_info	21

2.1.20 show anonymitylist_info	22

2.2. set	22

2.2.1 set analog_alarm	22

2.2.2 set cbat_pvid (portid) (pvid)	23

2.2.3 set cbat_dot1q_vlan_mode (mode)	23

2.2.4set card_igmp_snooping	23

2.2.3 set card_isolation	24

2.2.4 set card_qos_dscp	24

2.2.5 set card_qos_queue_threshold	25

2.2.6 set card_qos_queue_ttl	25

2.2.7 set card_qos_service_pri	25

2.2.8 set card_qos_threshold	26

2.2.9 set card_qos_tos	26

2.2.10 set card_qos_tos_mapping	27

2.2.11 set card_qos_vlan	27

2.2.12 set card_qos_vlan_mapping	27

2.2.13 set cbat_bstorm_ctrl	28

2.2.14 set cbat_bstorm_rate	28

2.2.15 set cnu_bstorm_ctrl	29

2.2.16 set cnu_rt_stats_reset	29

2.2.17 set cnu_cable_speedlimit	29

2.2.18 set cnu_igmpsp_ctrl	30

2.2.19 set cnu_isolation	30

2.2.20 set cnu_maclimit	30

2.2.21 set cnu_mstorm_ctrl	31

2.2.22 set cnu_port_linkpolicy	31

2.2.23 set cnu_port_pri	32

2.2.24 set cnu_port_pvid	32

2.2.25 set cnu_port_status	33

2.2.26 set cnu_port_speedlimit	33

2.2.27 set cnu_qos_dscp	33

2.2.28 set cnu_qos_vlan	34

2.2.29 set cnu_qos_queue_threshold	34

2.2.30 set cnu_qos_queue_ttl	34

2.2.31 set cnu_qos_service_pri	35

2.2.32 set cnu_qos_threshold	35

2.2.33 set cnu_qos_tos	36

2.2.34 set cnu_qos_tos_mapping	36

2.2.35 set cnu_qos_vlan_mapping	37

2.2.36 set cnu_stormctrl_pps	37

2.2.37 set cnu_vlanmode	37

2.2.38 set cnu_unkstorm_ctrl	38

2.2.39 set dns_server	38

2.2.40 set ipaddr	39

2.2.41 set ntp_server	39

2.2.42 set mgmt_vlan	40

2.2.43 set trap_server	40

2.2.44 set upgrade_server	41

2.2.45 set default_cnu_igmpsp_ctrl	41

2.2.46 set default_cnu_bstorm_ctrl	41

2.2.47 set default_cnu_mstorm_ctrl	42

2.2.48 set default_cnu_unkstorm_ctrl	42

2.2.49 set default_cnu_stormctrl_pps	42

2.2.50 set default_cnu_cable_speedlimit	43

2.2.51 set default_cnu_port_speedlimit	43

2.2.52 set default_cnu_vlanmode	43

2.2.53 set default_cnu_maclimit	44

2.2.54 set default_cnu_port_linkpolicy	44

2.2.55 set default_cnu_port_pri	44

2.2.56 set default_cnu_port_status	45

2.2.57 set default_port_pvid	45

2.2.58 set cnu_loopdetect_mode	45

2.3. add	46

2.3.1 add whitelist	46

2.4. del	46

2.4.1 del cnu	46

2.4.2 del white	47

2.5.  reset	47

2.5.1 reset card	47

2.5.2 reset cbat	48

2.5.3 reset cnu	48

2.6 restore	48

2.6.1  restore cbat_factory_default	48

2.6.2 restore cnu_factory_default	49

2.7. upgrade	49

2.7.1 upgrade app	49

2.7.2 upgrade kernel	50

3.Cablengine® MEoC command of users managment	50

3.1 keepalive	50

3.2. logout	50

3.3. who	51

# Cablengine® MEoC Configuration Environment

## 1.1 Configuration Environment

Fig.1.1 WEB management configuration environment

Confirm the connection between Master and Slave, then use Switch connect with Master, uplink to Computer, show as fig.1.1.

## 1.2 Steps of Configuration Environment

Setp 1：As shown fig.1-1,Connect PC ,Master by switch .

Master’s default network conig.（IP:192.168.0.254；subnet mask:255.255.255.0；default gateway:192.168.0.1；DNS is 61.177.7.1；NTP server is time.nist.gov）；

Step 2：[Start→Run→cmd→telnet Master IP Address 23→enter user ID and password( root、public)

Step 3：enter command，Configure equipment or check the equipment running status, Please got the help from(TAB) if it is necessary

## 1.3 Note

1, the MEoC management for for common computer or server;

2, system memory requirements above 128 m;

3, Operating system support  Windows 2000, Windows XP, Linux, Mac OS X operating system

2.Cable Speed M240NR  Command description

# 2.1 show

### 2.1.1. show cbat_info

【Command function】

Check the system information

【command format】

show cbat_info

【user guide】

Datetime; the present time of master；

Starttime;the starting time of master；

Configuration；The network info.of master is automatically or static configuration

Software Version；indicate the software Version No.

Hardware Version；indicate the hardware version

StartFreq; Start Frequency

StopFreq;End Frequency

Workmode;OFDM/CMTS

Upgrade Server Indicate the IP address when upgrade by FTP or TFTP server

Upgrade Server Port ; Update server port number

Telnet timeout

Bstorm Mode; Broadcast Limit

Bstorm Rate; The broadcast storm inhibition rate,

Dns Server;Dns server’s IP,default is  Dns management ip.

Ntp Server   net；Only can set IP address,default is “time.nist.gov”。

Flow Control ;the status of flow,default is enable。

Dot1q Vlan Mode，Master’s vlan enable

【For example】

MEOC>>show cbat_info

Date Time           : 1970-01-02 18:59:30

Start Time          : 1970-01-01 00:00:27

Cpu Mac             : 00:55:ff:01:00:01

Tech Project        : HomePlugAV

Model Number        : M240NR

Configuration       : Static Ip Address

Software Version    : 5.b.0.8

Hardware Version    : M2400NR-REV1.0

Ip Address          : 192.168.1.67

Netmask             : 255.255.255.0

Gateway             : 192.168.1.1

Start Freq          : 7.5 MHz

Stop Freq           : 67.5 MHz

Work Mode           : OFDM

Upgrade Server      : 192.168.0.122

Upgrade Server Port : 21

Dns Server          : 192.168.0.1

Ntp Server          : time.nist.gov

Max CnuCount        : 64

Max CnuOnline       : 64

Scan Mode           : Disable

Flow Control        : Enable

BStorm Mode         : Disable

BStorm Rate         : 1 per mill of port link rate

Dot1q Vlan Mode     : Disable

------------------------------------

Cbat Port Info

Port 1:

Status     : Deactive

Pvid       : 1

Port 2 (Management Port):

Status     : Active

LinkPolicy : 100M Full Duplex

Pvid       : 1

### 2.1.2. show cbat_topo_info

【For example】

MEOC>>show cbat_topo_info

Cpu Mac    : 00:55:ff:01:00:01

Card Count : 1

Id       Mac Address             Status

-----------------------------------------

0        00:55:ff:00:00:05       Reg Ok

【related command】

无。

### 2.1.3. show cbat_perf

【command function】

显示当前局端的性能信息

【command format】

show cbat_perf

【parameter description】

无。

【使用指南】

Cpu Load 表示cpu的负载；

Memory Usage 表示内存使用率

【For example】

MEOC>>show cbat_perf

Cpu Load     : 20%

Memory Usage : 50%

【related command】

无。

### 2.1.4 show cbat_vlan_list

【For example】

MEOC>>show cbat_vlan_list

VLAN List: 1, 11, 20, 400, 1200,

### 2.1.5 show cbat_vlan_info (vlanid)

【For example】

MEOC>>show cbat_vlan_info 11

VLAN: 11

Member Ports: 1, 2,

Untag Member Ports: 1, 2,

### 2.1.6 show cbat_rt_stats

### 【For example】

MEOC>>show cbat_rt_stats

Port 1:

Tx Bytes             : 0

Tx Unicast Packets   : 0

Tx Broadcast Packets : 0

Tx Multicast Packets : 0

Rx Bytes             : 0

Rx Unicast Packets   : 0

Rx Broadcast Packets : 0

Rx Multicast Packets : 0

Rx Dropped Packets   : 0

Port 2 (Management Port):

Tx Bytes             : 360795

Tx Unicast Packets   : 1405

Tx Broadcast Packets : 0

Tx Multicast Packets : 0

Rx Bytes             : 1734072

Rx Unicast Packets   : 1415

Rx Broadcast Packets : 19765

Rx Multicast Packets : 646

Rx Dropped Packets   : 0

### 2.1.7 show card_info

【user guide】

Firstly need use show cbat_topo_info to check the S/N.No.It is the Id.

【For example】

MEOC>>show card_info 0

Card Id          : 0

Card Mac         : 00:00:00:00:1a:06

Status           : Reg Ok

Online Time      : 1970-01-01 00:00:33

FW Version       : INT7400-MAC-7-0-7001-14-31-20110706-FINAL-D

Model Number     : M240NR

Isolation        : Enable

IgmpSp Mode      : Enable

Max Cnu          : 64

Cnu Count        : 3

Cnu Online Count : 3

Running Time     : 0d:0h:5m:48s

### 2.1.8 show card_topo_info

【command function】

Indicate card topo information

【command format】

show card_topo_info(cardid)

【parameter Description】

Param:Need show card’s S/N。

【For example】

MEOC>>show card_topo_info 0

Card Id    : 0

Card Mac   : 00:55:ff:00:00:05

Cnu Count  : 29

Id   Mac Address        Status   Access   Authorize   Tx   Rx

---------------------------------------------------------------

6    00:55:ff:00:00:46  Online   Deny     Yes         0    0

7    00:55:ff:00:00:47  Online   Deny     Yes         0    0

11   00:55:ff:00:00:4b  Online   Deny     Yes         0    0

21   00:55:ff:00:00:55  Online   Deny     Yes         0    0

22   00:55:ff:00:00:56  Online   Deny     Yes         0    0

26   00:55:ff:00:00:5a  Online   Deny     Yes         0    0

29   00:55:ff:00:00:5d  Online   Deny     Yes         0    0

无

### 2.1.9 show card_rf_info

【For example】

MEOC>>show card_rf_info 0

Card Id              : 0

Link MaxChannelNum   : 2452

Output Level         : 105 dBuV

DownStream StartFreq : 7.5 MHz

DownStream StopFreq  : 67.5 MHz

UpStream StartFreq   : 7.5 MHz

UpStream StopFreq    : 67.5 MHz

### 2.1.10 show card_qos

【For example】

MEoC>>show card_qos 0

card ID: 0

VLAN Qos: enable  TOS Qos: disable

Priority      VLAN Tags       TOS Bits

0             CA1             CA1

1             CA0             CA0

2             CA0             CA0

3             CA1             CA1

4             CA2             CA2

5             CA2             CA2

6             CA3             CA3

7             CA3             CA3

TTL:  CA0     CA1     CA2     CA3     MME

2000    2000       300     300      300

Default CAP:

IGMP                      : CA3

IGMP managed Multicast Stream: CA2

Multicast/Broadcast           : CA1

Unicast                     : CA1

TX Buffer Allocation: Enable

Allocation:   CA0     CA1     CA2     CA3

20%     25%     45%     10%

【related command】

show cnu_qos。

### 2.1.11 show analog_info

【For example】

MEOC>>show analog_info

AnalogType      HIHI      HI      LO      LOLO      Deadband

Temperature       80       60       0          0          0

CPU             100      60       0          0           0

Memory          100      60       0          0           0

Velocity          100      60       0          0           0

Yawp            100      60       0          0           0

Attenuation       100       60       0         0           0

### 2.1.12 show cnu_info

【command function】

显示终端的设备信息

【command format】

show cnu_info (cnuid)

【parameter description】

cnuid:要详细显示的cnu的序号。

【使用指南】

Authorize： authorization status

Access，allow，deny；

Port Isolation Mode:enable mean open，disable mean close。

Loop Detection Mode：enable，disable

Loop Status:looping，no loop

Phy Tx  cable

Phy Rx  cable

Phy Tx Link Quality

Phy Rx Link Quality

Avg Bits/Carrier

Avg SNR/Carrier

Avg Attenuation/Carrier

Avg Pre-FEC Bit Error Rate

【For example】

MEOC>>show cnu_info 6

Cnu 6

Cnu Mac      : 00:55:FF:00:00:46

Card Id      : 0

Init         : Finished

Online Time  : 1970-01-01 00:00:43

FW Version   : INT6000-MAC-4-1-4102-00-3679-20090724-FINAL-C

Custorm Type : s122prv

Switch Type  : Realtek

Port Num     : 2

Authorize    : Yes

Access       : Deny

Status       : Online

Isolation    : Enable

------------------------------------

Switch Info

IgmpSp Mode         : Disable

BStorm Mode         : Disable

MStorm Mode         : Disable

UnkStorm Mode       : Disable

Storm Rate          : 80 pps

Port Isolation Mode : Disable

Loop Detection Mode : Disable

Loop Status         : No Loop

Vlan Mode           : Transparent

Cable Up Limit      : 100 Mbps

Cable Dn Limit      : 100 Mbps

Mac Limit           : 0, Disable

------------------------------------

Link Info

Phy Tx                     : 0 Mbps

Phy Rx                     : 0 Mbps

Phy Tx Link Quality        : 0%

Phy Rx Link Quality        : 0%

Avg Bits/Carrier           : 0.00

Avg SNR/Carrier            : 0.00

Avg Attenuation/Carrier    : 0

Avg Pre-FEC Bit Error Rate : 0.00%

------------------------------------

Ethernet Port Info

LAN 1:

LinkPolicy    : Auto

Pri           : 0

Status        : Active

Pvid          : 1

Port Up Limit : 100 Mbps

Port Dn Limit : 100 Mbps

LAN 2:

LinkPolicy    : Auto

Pri           : 0

Status        : Active

Pvid          : 1

Port Up Limit : 100 Mbps

Port Dn Limit : 100 Mbps

### 2.1.13 show cnu_qos

【For example】

MEoC>>show cnu_qos 16

cnu ID: 16

VLAN Qos: enable  TOS Qos: disable

Priority      VLAN Tags       TOS Bits

0             CA1             CA1

1             CA0             CA0

2             CA0             CA0

3             CA1             CA1

4             CA2             CA2

5             CA2             CA2

6             CA3             CA3

7             CA3             CA3

TTL:  CA0     CA1     CA2     CA3     MME

2000    2000    300     300     300

Default CAP:

IGMP                      : CA3

IGMP managed Multicast Stream: CA2

Multicast/Broadcast           : CA1

Unicast                     : CA1

TX Buffer Allocation: Enable

Allocation:   CA0     CA1     CA2     CA3

20%     25%     45%     10%

【related command】

show card_qos.

### 2.1.14 show cnu_rt_stats

【For example】

MEOC>>show cnu_rt_stats 28

Cnu 28

Tx Packets           : 258051162

Rx Packets           : 12647621

Rx Droped Packets    : 0

Rx CRC Error Packets : 0

【related command】

show card_rt_stats

### 2.1.15 show default_cnu_config

【For example】

MEOC>>show default_cnu_config

Default Cnu Config:

Access: Deny

------------------------------------

Switch Info

IgmpSp Mode    : Disable

BStorm Mode    : Disable

MStorm Mode    : Disable

UnkStorm Mode  : Disable

Storm Rate     : 80 pps

Cable Up Limit : 100 Mbps

Cable Dn Limit : 100 Mbps

Vlan Mode      : Transparent

Mac Limit      : 0, Disable

------------------------------------

Ethernet Port Info

LAN 1:

LinkPolicy    : Auto

Pri           : 0

Status        : Active

Pvid          : 1

Port Up Limit : 100 Mbps

Port Dn Limit : 100 Mbps

LAN 2:

LinkPolicy    : Auto

Pri           : 0

Status        : Active

Pvid          : 1

Port Up Limit : 100 Mbps

Port Dn Limit : 100 Mbps

LAN 3:

LinkPolicy    : Auto

Pri           : 0

Status        : Active

Pvid          : 1

Port Up Limit : 100 Mbps

Port Dn Limit : 100 Mbps

LAN 4:

LinkPolicy    : Auto

Pri           : 0

Status        : Active

Pvid          : 1

Port Up Limit : 100 Mbps

Port Dn Limit : 100 Mbps

------------------------------------

Qos Info

Vlan Qos : Enable

Tos Qos  : Disable

Priority  Vlan Tags  Tos Bits

0         CA1        CA1

1         CA0        CA0

2         CA0        CA0

3         CA1        CA1

4         CA2        CA2

5         CA2        CA2

6         CA3        CA3

7         CA3        CA3

TTL

CA0    CA1    CA2    CA3    MME

2000   2000   300    300    300

Default CAP

Igmp                          : CA3

Igmp Managed Multicast Stream : CA2

Multicast/Broadcast           : CA1

Unicast                       : CA1

Tx Buffer Allocation : Disable

Allocation

CA0    CA1    CA2    CA3

20 %   25 %   45 %   10 %

### 2.1.16 show ip_access_list

【For example】

MEOC>>show ip_access_list

Ip Access Control: Disable

Ip Access List (Total Num: 1):

Ip: 192.168.1.60, Netmask: 255.255.255.255

### 2.1.17 show mgmt_vlan

【For example】

MEoC>>show mgmt_vlan

Management vlan :

mode:Disable

vlanid:0

### 2.1.18 show pre_cnu_list

### MEOC>>show pre_cnu_list

Id           Mac Address

--------------------------------

1            00:55:ff:00:21:03

2            00:55:ff:00:21:06

3            00:55:ff:00:21:05

4            00:55:ff:00:21:08

5            00:55:ff:01:21:05

### 2.1.19 show pre_cnu_config

【For example】

MEOC>>show pre_cnu_config 00:55:ff:01:21:05

Pre Cnu Config:

------------------------------------

Switch Info

IgmpSp Mode    : Disable

BStorm Mode    : Enable

MStorm Mode    : Disable

UnkStorm Mode  : Disable

Storm Rate     : 80 pps

Cable Up Limit : 100 Mbps

Cable Dn Limit : 100 Mbps

Vlan Mode      : Access

Mac Limit      : 0, Disable

------------------------------------

Ethernet Port Info

LAN 1:

LinkPolicy    : Auto

Pri           : 0

Status        : Active

Pvid          : 1

Port Up Limit : 100 Mbps

Port Dn Limit : 100 Mbps

LAN 2:

LinkPolicy    : Auto

Pri           : 0

Status        : Active

Pvid          : 1

Port Up Limit : 100 Mbps

Port Dn Limit : 100 Mbps

LAN 3:

LinkPolicy    : Auto

Pri           : 0

Status        : Active

Pvid          : 1

Port Up Limit : 100 Mbps

Port Dn Limit : 100 Mbps

LAN 4:

LinkPolicy    : Auto

Pri           : 0

Status        : Active

Pvid          : 1

Port Up Limit : 100 Mbps

Port Dn Limit : 100 Mbps

### 2.1.20 show trap_server

【user guide】

Max Times range <1-255>

Min Interval range<1-255>

Max Interva range <1-255>

【For example】

MEOC>>show trap_server

Ip Address   : 192.168.1.55

Max Times    : 255

Min Interval : 0

Max Interval : 0

Community    : public

Status       : Enable

【related command】

set trap_server

### 2.1.21 show whitelist_info

【For example】

MEOC>>show whitelist_info

Register WhiteList Number : 3

All RF Output Level       : 105 dBuV

Cnu Mac Limit Number      : 8

Phy Rate Gate             : 200000000 bps

Cnu 8 Info

Card Id           : 0

Cnu Mac           : f4:3e:61:da:19:c8

Cnu Status        : Online

Cnu 28 Info

Card Id           : 0

Cnu Mac           : f4:3e:61:da:af:dc

Cnu Status        : Online

Cnu 60 Info

Card Id           : 0

Cnu Mac           : f4:3e:61:da:19:bc

Cnu Status        : Online

【related】

add white_mac和del white_mac.

### 2.1.22 show anonymitylist_info

【command function】

Show MAC black list in the system

## 2.2. set

### 2.2.1 set analog_alarm

【command format】

set analog_alarm (analog)  (hihi)  (hi)  (lo)  (lolo)  (deadband)

【parameter specification】

Analog items:,

1 – Temperature

2 – CPU         cpu Utilization Rate

3 – memory      memory Utilization Rate

4 – velocity 		link rate

5 – yawp        Noise

6 – attenuation

hihi : too high

hi : a little high

lo : a little low

lolo : too low

deadband :dead zone value

【For example】

MEOC>>set analog_alarm 1 70 50 20 10 10

Set Analog Success.

### 2.2.2set card_igmp_snooping

【command formart】

set card_igmp_snooping (cardid) (mode)

【parameter description】

mode: enable mean open

disable mean close。

【For example】

MEoC>>set card_igmp_snooping 0 enable

Set Card 0 IGMP Snooping Success.

### 2.2.3 set card_isolation

【command format】

set card_isolation (cardid) (mode)

【parameter description】

mode:enable

disable

【For example】

MEoC>>set card_isolation 0 disable

Set Card 0 Isolation Success.

### 2.2.4 set card_qos_dscp

【command format】

set card_qos_dscp (cardid) (mode)

【parameter description】

mode:enable

disable

【For example】

MEoC>>set card_qos_dscp 0 enable

Set Card 0 QoS Dscp enable Success.

【related command】

show card_qos

### 2.2.5 set card_qos_queue_threshold

### 【command format】

set card_qos_queue_threshold (cardid) (queue) (percent)

【parameter description】

queue:CA1～CA3

percent:

【For example】

MEoC>>set card_qos_queue_threshold 0 CA1 40

Set QoS Queue Threshold Success.

【related command】

show card_qos

### 2.2.6 set card_qos_queue_ttl

【command format】

set card_qos_queue_ttl(cardid) (queue) (ttl)

【parameter description】

queue: CA0~CA3,MME

ttl: 10~65000

Ttl MME range is  300～2000

【For example】

MEoC>> set card_qos_queue_ttl 0 CA2 300

Set Card 0 QoS TTL Success.

【related command】

show card_qos

### 2.2.7 set card_qos_service_pri

【command format】

set card_qos_service_pri (cardid) (type) (queue)

【parameter description】

type：unicast

mcast

igmp

avstream  IGMP Controlled multicast

【For example】

MEoC>> set card_qos_service_pri 0 mcast CA1

Set Card 0 QoS Service Priority Success.

【related command】

show card_qos

2.2.8 set card_qos_threshold

【command format】

set card_qos_threshold (cardid) (operation)

【parameter description】

operation: enable

disable

【For example】

MEoC>> set card_qos_threshold 0 enable

Set Card 0 QoS Threshold Enable Success.

【related command】

show card_qos

2.2.9 set card_qos_tos

【command function】

Enable or disable qos and tos function

【command format】

set card_qos_tos (cardid) (operation)

【parameter description】

param: enable；

disable 。

【For example】

MEoC>> set card_qos_tos 0 enable

Set Card 0 QoS Tos enable Success.

【related command】

### 2.2.10 set card_qos_tos_mapping

【command format】

set card_qos_tos_mapping (cardid) (queue) (tos)

【parameter description】

queue: CA0~CA3

tos:0~7。

【user gudie】

CA3 is the highest，CA0 is the lowest

【For example】

MEoC>> set card_qos_tos_mapping 0 CA2 4

Set Card 0 QoS Tos Mapping Success.

【related command】

### 2.2.11 set card_qos_vlan

【command function】

enable or disable qos’s vlan function

【command format】

set card_qos_vlan (cardid) (operation)

【parameter description】

param: enable ；

disable。

【For example】

MEoC>> set card_qos_vlan 0 enable

Set Card 0 QoS Vlan enable Success.

【related command】

show card_qos

### 2.2.12 set card_qos_vlan_mapping

【command function】

Set the mapping between qos and vlan

【command format】

set card_qos_vlan_mapping (cardid) (queue) (vlanid)

【parameter description】

queue CA0~CA3:

【user guide】

CA3 is the highest，CA0 is the lowesst

【For example】

MEoC>> set card_qos_vlan_mapping 0 CA3 3

Set Card 0 QoS Vlan Mapping Success.

【related command】

show card_qos

### 2.2.13 set cbat_rt_stats_reset

【command function】

Remove the card real-time performance information

【command format】

set cbat_rt_stats_reset

【parameter description】

【For example】

MEOC>>set cbat_rt_stats_reset

Clean Cbat Realtime Statistic Success

### 2.2.14 set cbat_pvid (portid) (pvid)

【command function】

Set port pvid

【command format】

set cbat_pvid (portid) (pvid)

【parameter description】

Portid：cbat port No.,range<1~2>；pvid：cbat’s port vid range is<1~4094>；

【For example】

MEOC>>set cbat_pvid 2 200

【related command】

### 2.2.15 set cbat_dot1q_vlan_mode (mode)

【command function】

enable or disable802.1q vlan功能

【command format】

set cbat_dot1q_vlan_mode (mode)

【parameter description】

mode(enable|disable)

【For example】

【related command】

### 2.2.16 set cbat_bstorm_ctrl

【command function】

enable or disable master’s broadcast limit function

【command format】

set cbat_bstorm_ctrl (mode)

【parameter description】

mode：enable；disable 。

【For example】

MEoC>> set cbat_bstorm_ctrl enable

Set Cbat Broadcast Storm Control Enable Success.

【related command】

### 2.2.17 set cbat_bstorm_rate

【command format】

cbat_bstorm_rate

【For example】

MEOC>>set cbat_bstorm_rate 10

【related command】

### 2.2.18 set cbat_flow_ctrl

【command format】

set cbat_flow_ctrl (mode)

【parameter description】

Mode：enable|disable；

Enable

Disable

【For example】

MEOC>>set cbat_flow_ctrl disable

【related command】

### 2.2.19 set cbat_max_cnu_count

【For example】

MEOC>> Set cbat_max_cnu_count 10

### 2.2.20 set cnu_bstorm_ctrl

### 【For example】

MEOC>> set cnu_bstorm_ctrl 16 enable

Set Cnu:16 Broadcast Storm Control Enable Success.

【related command】

### 2.2.21 set cnu_rt_stats_reset

【command function】

Remove the slave’s real-time performance information

【command format】

set cnu_rt_stats_reset (cnuid)

【For example】

MEOC>>set cnu_rt_stats_reset 28

Clean Cnu :28 Realtime Statistic Success.

### 2.2.22 set cnu_cable_speedlimit

【command function】

Set the uplink and downlink bandwidth of cnu

【command format】

set cnu_cable_speedlimit (cnuid) (uplink) (downlink)

【parameter description】

uplink

downlink

【For example】

MEOC>>set cnu_cable_speedlimit 24 1 16

Set Cnu:24 Speed limited Success.

### 2.2.23 set cnu_igmpsp_ctrl

【command function】

enable or disable cnu’s igmp snooping

【command format】

set cnu_igmpsp_ctrl (cnuid) (mode)

【parameter description】

mode：enable | disable

【For example】

MEOC>> set cnu_igmpsp_ctrl 16 enable

Set Cnu:16 IGMP Snooping Control Enable Success.

### 2.2.24 set cnu_loopdetect_mode

【command function】

enable or disable

【command format】

set cnu_loopdetect_mode (cnuid)  (mode)

【parameter description】

Cnuid：cnu id

mode：enable|disalbe

【For example】

MEOC>>set cnu_loopdetect_mode 16 enable

Set Cnu:16 Loop Detection Enable Success.

### 2.2.25 set cnu_isolation

### 【For example】

MEOC>> set cnu_isolation 16 disable

Set Cnu:16 Isolation Success.

### 2.2.26 set cnu_maclimit

【command function】

Set cnu max qty of access

【command format】

set cnu_maclimit (cnuid) (macnum)

【parameter description】

The qty of macnum is [0,8]

【For example】

MEOC>> set cnu_maclimit 16 5

Set Cnu:16 Mac Limit  Success.

### 2.2.27 set cnu_mstorm_ctrl

【For example】

MEOC>>set cnu_mstorm_ctrl 28 enable

Set Cnu:28 Multicast Storm Control Enable Success.

### 2.2.28 set cnu_port_linkpolicy

【command function】

Set cn port workmode

【command format】

set cnu_port_linkpolicy (cnuid) (port) (linkpolicy)

【parameter description】

linkpolicy：auto

100_full

100_half

10_full

10_half

【For example】

MEoC>> set cnu_port_linkpolicy 16 1 auto

Set Cnu 16 Port 1 Linkpolicy Success.

### 2.2.29 set cnu_port_pri

【For example】

MEoC>> set cnu_port_pri 16 1 4

Set Cnu:16 Port 1 Pri 4 Success.

### 2.2.30 set cnu_port_pvid

【For example】

MEOC>> set cnu_port_pvid 16 1 5

Set Cnu:16 Port 1 Vlan Success.

【related command】

无。

### 2.2.31 set cnu_port_status

【For example】

MEOC>> set cnu_port_status 16 1 open

Set Cnu:16 Port 1 status open Success.

### 2.2.32 set cnu_port_speedlimit

【command function】

Set cnu port speedlimit

【command format】

set cnu_port_speedlimit (cnuid) (port) (upspeed) (downspeed)

【parameter description】

Cnuid：cnu’s ID no.

Port：cnu port NO.

Upspeed：uplink

Downspeed：downlink

【For example】

MEOC>>set cnu_port_speedlimit 24 1 5 5

Set Cnu:24 Port 1 Speedlimit Success.

### 2.2.33 set cnu_port_isolation_mode

【For example】

MEOC>> set cnu_port_isolation_mode 31 enable

【related command】

### 2.2.34 set cnu_qos_dscp

【For example】

MEOC>> set cnu_qos_dscp 16 enable

Set Cnu 16 QoS Dscp Enable Success.

【related command】

无。

### 2.2.35 set cnu_qos_vlan

【For example】

MEOC>> set cnu_qos_vlan 16 enable

Set Cnu 16 QoS Vlan Enable Success.

### 2.2.36 set cnu_qos_queue_threshold

【command function】

Set the percent of  priority queue in  intellon chip【command format】

set cnu_qos_queue_threshold (cnuid) (queue) (percent)

【parameter description】

queue：CA0～CA3

percent:

【user guide】

CA3 is the highest，CA0 is the lowest（unable be allowed to be set）

【For example】

MEoC>> set cnu_qos_queue_threshold 16 CA1 25

Set Cnu 16 QoS Queue Threshold Success.

### 2.2.37 set cnu_qos_queue_ttl

【For example】

MEoC>> set cnu_qos_queue_ttl 16 CA1 3000

Set Cnu 16 QoS TTL Success.

### 2.2.38 set cnu_qos_service_pri

【For example】

MEoC>> set cnu_qos_service_pri 16 mcast CA1

Set Cnu 16 QoS Service Priority Success.

### 2.2.39 set cnu_qos_threshold

【For example】

MEoC>> set cnu_qos_threshold 16 enable

Set Cnu 16 QoS Threshold Enable Success.

### 2.2.40 set cnu_qos_tos

【command function】

enable or disablecnu的qos tos

【command format】

set cnu_qos_tos (cnuid) (operation)

【parameter description】

operation:enable； disable。

【For example】

MEoC>> set cnu_qos_tos 16 enable

Set Cnu 16 QoS Tos Enable Success.

### 2.2.41 set cnu_qos_tos_mapping

【For example】

MEoC>>set cnu_qos_tos_mapping 16 CA1 4

Set Cnu 16 QoS Tos Mapping Success.

### 2.2.42 set cnu_qos_vlan_mapping

【For example】

MEoC>> set cnu_qos_vlan_mapping 16 CA2 7

Set Cnu 16 QoS Vlan Mapping Success.

### 2.2.43 set cnu_stormctrl_pps

【For example】

MEOC>>set cnu_stormctrl_pps 28 10

Set Cnu:28 Bstorm PPS Success.

### 2.2.44 set cnu_vlanmode

【For example】

MEoC>> set cnu_vlanmode 16 access

Set cnu:16 Vlan Mode Success.

### 2.2.45 set cnu_unkstorm_ctrl

【For example】

MEoC>> set cnu_unkstorm_ctrl 16 enable

Set Cnu:16 Unknown Storm Control Enable Success.

### 2.2.46 set dns_server

【For example】

MEOC>>set dns_server 192.43.244.18

Set Ntp Server Success.

### 2.2.47 set ipaddr

【command function】

Set system’s ip

【command format】

set ipaddr (ipconfig) (ip) (netmask) (gateway)

【parameter description】

ipconfig: static-> IP

dhcp-> IP

ip

netmas k

gateway

【For example】

Static

MEoC>> set ipaddr static 192.168.0.247 255.255.255.0 192.168.0.1

/*******************************caution**************************/

/*If you want to take effect the IP address,please Reset system*/

/****************************************************************/

Dhcp.

MEOC>>set ipaddr dhcp

Set Cbat Ip Success.

/*******************************caution**************************/

/*If you want to take effect the IP address, please Reset Cbat!*/

/****************************************************************/

### 2.2.48 set ip_access_ctrl

【command format】

set ip_access_ctrl (enable|disable)

### 2.2.49 set ntp_server

【For example】

MEOC>>set ntp_server 192.43.244.18

Set Ntp Server Success.

### 2.2.50 set mgmt_vlan

【For example】

MEoC>> set mgmt_vlan disable 8

/*******************************caution**************************/

/****Take effect the management vlan config,Need Reset Cbat****/

/****************************************************************/

### 2.2.51 set trap_server

【command function】

Set trapserver’s information。

【command format】

set trap_server (serverip) (maxtimes) (mininterval) (maxinterval) (community) (status)

【parameter description】

serverip：ip。

maxtimes: range at <0~255>

mininterval: <0~255>

maxinterval: <0~255>

community.

status:enable|disable

【For example】

MEOC>> set trap_server 192.168.0.125 200 100 3000 public enable

Set TrapServer Config Success.

【related command】

Show trap_server

### 2.2.52 set upgrade_server

【command function】

设置升级服务器ip。

【command format】

set upgrade_server (serverip) (port) (user) (password)

【parameter description】

Serverip:p。

port

use

password。

【user guide}

show cbat_info to check whether upgrade sucessfully

【For example】

MEOC>> set upgrade_server 192.168.0.9 21 root public

Set Upgrade Server Success.

### 2.2.53 set default_cnu_igmpsp_ctrl

【command format】

set default_cnu_igmpsp_ctrl (mode)

【parameter description】

mode : enable

disable

### 2.2.54 set default_cnu_bstorm_ctrl

。

【command format】

set default_cnu_bstorm_ctrl (mode)

【parameter description】

mode : enable

disable

【For example】

### 2.2.55 set default_cnu_mstorm_ctrl

【command format】

set default_cnu_mstorm_ctrl (mode)

【parameter description】

mode : enable  --

disable  –

### 2.2.56 set default_cnu_unkstorm_ctrl

【command format】

set default_cnu_unkstorm_ctrl (mode)

【parameter description】

mode : enable

disable

### 2.2.57 set default_cnu_stormctrl_pps

【command format】

set default_cnu_stormctrl_pps (number)

【parameter description】

number : 640,320,160,80,40,20,10

### 2.2.58 set default_cnu_cable_speedlimit

【command format】

set default_cnu_cable_speedlimit (upspeed) (downspeed)

### 2.2.59 set default_cnu_port_speedlimit

【command format】

set default_cnu_port_speedlimit (port) (upspeed) (downspeed)

【parameter description】

### 2.2.60 set default_cnu_vlanmode

【command format】

set default_cnu_vlanmode (mode)

【parameter description】

mode : transparent – ,disable vlan

access – enable vlan

### 2.2.61 set default_cnu_maclimit

【command format】

set default_cnu_maclimit (number)

### 2.2.62 set default_cnu_port_linkpolicy

【command format】

set default_cnu_port_linkpolicy (port) (linkpolicy)

【parameter description】

linkpolicy : auto

100_full

100_half

10_full

10_half

### 2.2.63 set default_cnu_port_pri

【command format】

set default_cnu_port_pri (port) (pri)

【parameter description】

pri :  0~7

### 2.2.64 set default_cnu_port_status

【command format】

set default_cnu_port_status (port) (status)

【parameter description】

status : open –

close –

### 2.2.65 set default_cnu_port_pvid

【command format】

set default_cnu_port_pvid  (port)  (pvid)

【parameter description】

pvid : 1~4094

2.2.66 Set default_cnu_access

【command format】

Set default_cnu_access (operation)

【parameter description】

operation : access|deny

【user guide】

Access：allow to access

Deny：refuse to access

## 2.3. add

### 2.3.1 add cbat_vlan (vlanid)

【command function】

Creat vlan。

【command format】

add cbat_vlan (vlanid)

【parameter description】

Vlanid:取值范围<1~4094>

### 2.3.2 add cbat_vlan_member (vlanid) (portid) (mode)

【command function】

Add vlan member and set the rules

【command format】

add cbat_vlan_member (vlanid) (portid) (mode)

【parameter description】

Vlanid:<1~4094>；

Portid:<1~2>；

Mode: (tag|untag)；tag is por’s pvid value，untag will be showed in cbat_vlan_info

【For example】

MEOC>>add cbat_vlan_member 1 1 untag

【related command】

MEOC>>show cbat_vlan_info 1

Vlan: 1

Member Ports: 1, 2,

Untag Member Ports: 1, 2,

### 2.3.3 add ip_access_allow

【For example】

MEOC>>add ip_access_allow 192.168.1.60 255.255.255.255

Add Ip Access Allow Success.

【related command】

### 2.3.4 add whitelist

【For example】

MEOC>>add whitelist 28

Add White Mac Success.

【related command】

show whitelist_info。查看

## 2.4. del

### 2.4.1 del cbat_vlan (vlanid)

【command function】

Delete vlan

【command format】

del cbat_vlan (vlanid)

【parameter description】

Vlanid：<1~4094>；

### 2.4.2 del cbat_vlan_member (vlanid) (portid)

【command function】

Delete vlan members

【command format】

del cbat_vlan_member (vlanid) (portid)

【parameter description】

Vlanid：<1~4094>；

Portid：<1~2>;

### 2.4.3 del cnu

【command function】

Delete slaves。

终端离线状态分为2种，

1  offlining

2  offline

【command format】

del cnu (cnuid)

【parameter description】

【For example】

MEoC>> del cnu 41

Del Cnu 41 Success.

MEOC>>del cnu 41

ERROR :Cnu Offlining, Try Later

ERROR :Ceserver Return Error!

### 2.4.4 del ip_access_allow

【command format】

ip_access_allow (ip)  (netmask)

【parameter description】

Ip：ip

Netmask：

【For example】

MEOC>>del ip_access_allow 192.168.0.0 255.255.255.255

Del Ip Access Allow Success.

### 2.4.5 del white

【command function】

Delete mac white list

【command format】

del whitelist (cnuid)

【For example】

MEOC>>del whitelist 28

Del White Mac Success

【related command】

show anonymitylist_info

## 2.5.  reset

### 2.5.1 reset card

【command function】

Reset card’s intellon chip

【command format】

reset card (cardid)

【For example】

MEoC>> reset card 2

Restart Card Success.

。

### 2.5.2 reset cbat

【command function】

Reset cbat

【command format】

reset cbat

【For example】

MEoC>> reset cbat

Reset Cbat Success.

### 2.5.3 reset cnu

【command function】

Reset cnu’s intellon chip

【command format】

reset cnu (cnuid)

【For example】

MEoC>> reset cnu 5

Reset Cnu 5 Success

【related command】

无。

## 2.6 restore

### 2.6.1  restore cbat_factory_default

【For example】

MEoC>> restore cbat_factory_default

Restore Cbat Factory Default Configuration Success

### 2.6.2 restore cnu_factory_default

【For example】

MEoC>> cnu_factory_default 2

Restore Cnu 2 Config Success.

。

## 2.7. upgrade

### 2.7.1 upgrade app

【command format】

upgrade app

### 2.7.2 upgrade kernel

【command function】

Upgrade master’s software-kernel

【command format】

upgrade kernel (kerne)

# 3.Cablengine® MEoC

## 3.1 keepalive

【For example】

MEOC>>keepalive 200

Set Telnet Keepalive Timer Success

MEOC>>keepalive

Telnet Keepalive Timer : 300s

【related command】

## 3.2. logout

【command function】

Quit CLI interface

【command format】

logout

【For example】

MEoC>>logout

## 3.3. who

【command format】

who

【For example】

MEoC>>who

+------------------------------------------------------+

| User Name | Login Channel | Login time    |

+------------------------------------------------------+

| root      | TELNET     | Wed Apr 22 20:51:50 2009

+------------------------------------------------------+